Network Traffic Analysis Report

Prepared by: Hala Mohamed Fouad

Date: April 2026

Tool Used: Wireshark 4.6.4

# 1. Introduction

This report presents the findings of a network traffic analysis conducted on a home Wi-Fi network using Wireshark. The main objective was to capture live network packets, examine the types of traffic being transmitted, and identify any patterns or behaviors that could pose a potential security risk. The analysis was carried out on a Windows 11 laptop connected to a private home network, and all captured data belongs entirely to that device and its communications.

The results documented here cover DNS activity, HTTP traffic, TCP session data, protocol distribution statistics, and endpoint communication summaries. Based on these findings, a set of practical recommendations has been provided to help improve the overall security posture of the network.

# 2. Tools and Environment

| Component | Details |
| --- | --- |
| Capture Tool | Wireshark 4.6.4 |
| Operating System | Windows 11 |
| Network Type | Home Wi-Fi (Private) |
| Network Interface | Wi-Fi Adapter |
| Capture File | network_capture.pcap |
| Total Packets Captured | 30,483 packets |
| Packets Displayed (TCP filter) | 13,495 (44.3%) |

# 3. Findings

## 3.1 DNS Traffic Analysis

Applying the dns filter in Wireshark revealed 564 DNS packets, making up roughly 1.9% of the total capture. The DNS queries were all sent from the local machine (192.168.1.18) to the DNS resolver at 163.121.128.134. The destinations being resolved were almost entirely Google-related services, which is expected given the browsing activity performed during the capture session.

The following domains were among those actively queried:

- safebrowsing.googleapis.com - Google Safe Browsing service running in the background

- play.google.com - Google Play services communicating with the device

- googleusercontent.com and l3.googleusercontent.com - CDN content delivery nodes

- gstatic.com and ssl.gstatic.com - Static assets and SSL certificate validation

- ogs.google.com - Google One service communications

One important observation here is that all DNS queries were sent in plaintext with no encryption layer. This is the standard behaviour for DNS unless DNS over HTTPS (DoH) or DNS over TLS (DoT) is configured. In practical terms, this means anyone on the same network segment can observe which domains the device is resolving, which is a privacy concern even if the actual content of the connections is encrypted.

## 3.2 HTTP Traffic Analysis

Filtering by http showed 14 HTTP packets in total, accounting for just 0.1% of all captured traffic. Despite the small count, these packets are worth examining because they travel without any encryption. The destination IP for all observed HTTP traffic was 92.223.47.61, and the request paths pointed to Microsoft Windows Update infrastructure, specifically certificate trust list files under /msdownload/update/v3/static/trustedr/.

All HTTP responses returned a 304 Not Modified status, meaning the client already had the files cached and no sensitive data was actually downloaded over the unencrypted channel. However, the fact that the connection was initiated over HTTP rather than HTTPS still represents a potential risk in adversarial environments. An attacker performing a man-in-the-middle attack on the same network could potentially intercept or modify these requests before the client verifies the certificate chain.

## 3.3 TCP Traffic Analysis

The TCP filter showed 13,495 packets out of the full capture. Looking at the packet details, the majority of TCP sessions were associated with TLS-encrypted connections, which is a positive sign. Cloudflare infrastructure (35.190.80.1) appeared frequently as a destination, reflecting typical web browsing to HTTPS websites that use Cloudflare's CDN.

A small number of TCP Retransmissions and Duplicate ACK packets were also visible in the capture. These are not inherently a security concern but they do suggest minor network instability or packet loss occurring at some points during the session. In a security context, an unusually high volume of retransmissions could also be a symptom of network congestion caused by a DoS-type event, though that was not the case here.

## 3.4 Protocol Hierarchy Statistics

The protocol hierarchy view gave a clear breakdown of what protocols were in use across the entire capture:

| Protocol | % of Packets | Packets | Notes |
| --- | --- | --- | --- |
| Frame / Ethernet | 100% | 13,495 | All captured traffic |
| Internet Protocol Version 4 (IPv4) | 100% | 13,495 | All traffic uses IPv4 |
| Transmission Control Protocol (TCP) | 100% | 13,495 | All sessions are TCP-based |
| Transport Layer Security (TLS) | 24.8% | 3,342 | Encrypted HTTPS traffic |
| Hypertext Transfer Protocol (HTTP) | 0.1% | 14 | Small amount of unencrypted traffic |
| Data | 1.2% | 160 | Raw data packets |

The high percentage of TLS traffic relative to raw HTTP is encouraging and shows that the majority of the network communications observed were encrypted. The absence of UDP-based protocols like DNS in the TCP-filtered view is expected since the DNS filter was applied separately.

## 3.5 Endpoint Communication Summary

The Endpoints window identified three active MAC addresses communicating on the network during the capture period:

| MAC Address | Total Packets | Total Bytes | Tx Packets | Rx Packets |
| --- | --- | --- | --- | --- |
| 14:b5:cc:13:7f:6d | 13,495 | 11 MB | 4,607 | 8,888 |
| a8:02:db:f5:48:94 | 13,483 | 11 MB (10 MB Rx) | 8,876 | 4,607 |
| b0:a7:b9:b0:97:af | 12 | 4 KB | 12 | 0 |

The first two MAC addresses account for almost all of the traffic exchanged and appear to be the laptop's own network adapter and the home router/gateway. The third address with only 12 packets and 4 KB of data is likely a secondary device on the network that briefly communicated during the capture window. No unfamiliar or suspicious external MAC addresses were identified.

At the IPv4 level, the major destinations observed were Google infrastructure IPs, Cloudflare nodes, and Microsoft update servers. All of these are expected and legitimate for a standard Windows machine with an active browser session.

# 4. Identified Security Concerns

| # | Issue | Protocol | Risk Level | Description |
| --- | --- | --- | --- | --- |
| 1 | Unencrypted DNS Queries | DNS | Medium | All DNS lookups sent in plaintext - visible to anyone on the network |
| 2 | HTTP Traffic to Microsoft Servers | HTTP | Low-Medium | Certificate trust list fetched over HTTP instead of HTTPS |
| 3 | TCP Retransmissions Observed | TCP | Low | Minor packet loss suggesting occasional network instability |
| 4 | Multiple External IP Endpoints | TCP/TLS | Informational | Device contacted Google, Cloudflare, and Microsoft IPs - all legitimate |

# 5. Recommendations

Based on the findings above, the following steps are recommended to strengthen the security of the network:

- Enable DNS over HTTPS (DoH): Configuring the browser or operating system to use an encrypted DNS resolver such as Cloudflare (1.1.1.1) or Google (8.8.8.8) with DoH enabled will prevent third parties from observing DNS queries.

- Prefer HTTPS-only Browsing: Most modern browsers offer an HTTPS-only mode that can be enabled in settings. This forces connections to use encryption wherever it is available and alerts the user if a site only supports HTTP.

- Use a VPN on Untrusted Networks: While the home network observed here is private, it is good practice to use a reputable VPN when connected to public Wi-Fi. A VPN encrypts all outgoing traffic and hides DNS queries from local observers.

- Monitor for Unusual Endpoints: Periodically reviewing the Endpoints list in Wireshark or a similar tool can help detect unexpected connections to unfamiliar IP addresses, which might indicate background processes communicating with unauthorised servers.

- Keep Windows Update Running: The HTTP connections observed were to Windows Update certificate infrastructure. Ensuring updates are applied regularly reduces the chance of exploitable vulnerabilities remaining on the system.

# 6. Conclusion

The network traffic analysis carried out using Wireshark on this home network returned results that are largely expected for a standard Windows 11 machine during a regular browsing session. The majority of traffic was encrypted using TLS, which is a strong indicator of good baseline security practices in place.

The main concerns identified were the use of unencrypted DNS queries, which expose domain resolution activity to potential observers on the same network, and a small number of HTTP requests to Microsoft update servers. Neither of these represents an active threat in a private home network context, but both are areas where simple configuration changes could meaningfully reduce the attack surface.

No malicious traffic, suspicious external endpoints, or evidence of intrusion was detected during this capture session. With the recommended measures in place, the overall security of this network would be substantially improved.